#### Raindrop module (double panel)

Product Description.

1, the sensor is made of high quality FR-04 double-sided material, large area 5.0 * 4.0CM, and with nickel-plated surface, with antioxidant, conductivity, and the life of the more superior performance;

2, comparator output, clean signal, good waveform, drive ability, more than 15mA;

3, with potentiometer to adjust the sensitivity;

4、Operating voltage 3.3V-5V

5、Output form ：Digital switch output (0 and 1) and analog AO voltage output;

6, with fixed bolt holes, easy to install

7, small board PCB size: 3.2cm x 1.4cm

8, the use of wide voltage LM393 comparator

Function introduction:

Connected to the 5V power supply, the power indicator light, no water droplets on the sensor board, DO output is high, switch indicator light off , drop a drop of water, DO output is low, switch indicator light on.

Brush off the water drop above, and then return to, output high level state 。。。。

AO analog output, can be connected to the AD port of the microcontroller to detect the size of the rain drops on it.

DO TTL digital output can also be connected to the microcontroller to detect whether there is rain.

Wiring mode:

1、VCC: connected to the positive terminal of the power supply (3-5V)

2、GND:connect the negative pole of power supply.

3、DO:TTL switch signal output

4、AO:Analog signal output